IoT-Based Weather Station Using ESP32 and Cloud Dashboard

Project Report — Industrial Training Program on Robotics and AI

# 1. Problem Statement

This project designs and implements an IoT-based weather monitoring system built around an ESP32 microcontroller. The system:

- Reads temperature and humidity using a DHT22 sensor.

- Sends sensor data to the ThingSpeak cloud platform over the ESP32's built-in Wi-Fi.

- Displays live graphs of temperature and humidity on a ThingSpeak channel dashboard.

- Triggers an email alert whenever temperature or humidity crosses a predefined threshold.

# 2. System Architecture (Question 1)

The figure below shows the end-to-end data flow. The DHT22 sensor is wired to a GPIO pin on the ESP32. The ESP32 firmware polls the sensor at a fixed interval, connects to the local Wi-Fi network, and pushes each reading to a ThingSpeak channel over HTTP using a REST API write key. ThingSpeak stores the values as two channel fields (Field 1 = Temperature, Field 2 = Humidity) and renders them as live line graphs. A ThingHTTP / IFTTT Webhooks action, triggered either by a ThingSpeak React rule or directly by the firmware, sends an email whenever a value goes outside the safe range.

Figure 1: Block diagram of the IoT weather station

## Signal / data path

- DHT22 -> ESP32: single-wire digital data line (GPIO 4), sensor powered from 3.3 V.

- ESP32 -> Wi-Fi router: 2.4 GHz WPA2 station connection.

- ESP32 -> ThingSpeak: HTTPS/HTTP GET request to the Update API with the channel write key and two field values.

- ThingSpeak -> Browser: channel's public/private view renders Field 1 and Field 2 as auto-updating charts.

- ThingSpeak/ESP32 -> IFTTT Webhooks -> Email: alert path used for threshold notifications.

# 3. Sensor Interfacing Firmware (Question 2)

The DHT22 is read using the Adafruit DHT library, which handles the single-wire timing protocol. The core routine below returns both readings and validates them, since the DHT22 occasionally returns NaN on a noisy read:

#define DHT_PIN   4

#define DHT_TYPE  DHT22

DHT dht(DHT_PIN, DHT_TYPE);

bool readSensor(float &temperature, float &humidity) {

temperature = dht.readTemperature();  // Celsius

humidity    = dht.readHumidity();     // % RH

if (isnan(temperature) || isnan(humidity)) {

Serial.println("ERROR: Failed to read DHT22!");

return false;

}

return true;

}

Full firmware is provided in the accompanying source file weather_station.ino.

# 4. Cloud Data Upload (Question 3)

Every UPLOAD_INTERVAL (20 seconds, respecting ThingSpeak's free-tier 15-second minimum), the firmware builds an HTTP GET request to the ThingSpeak Update API, embedding the write API key and the two field values:

String url = String(TS_SERVER) + "?api_key=" + TS_API_KEY +

"&field1=" + String(temperature, 2) +

"&field2=" + String(humidity, 2);

http.begin(url);

int httpCode = http.GET();

## Verifying the upload

- ThingSpeak's GET response body is the new entry ID as an integer; a value of 0 means the update was rejected (e.g. rate-limited or wrong key).

- The Arduino IDE Serial Monitor (115200 baud) prints each reading and the returned entry ID for quick verification.

- The channel's "Private View" > "Data Import/Export" tab lists every logged entry with a timestamp, confirming both fields update correctly.

# 5. Data Visualization (Question 4)

On ThingSpeak, create a channel with two fields: Field 1 = Temperature (°C) and Field 2 = Humidity (%RH). Steps to configure live graphs:

- Sign in to thingspeak.com and select New Channel.

- Enable Field 1 and Field 2, name them Temperature and Humidity, and save the channel.

- Copy the channel's Write API Key into TS_API_KEY in the firmware.

- Open the channel's Private View — ThingSpeak automatically generates a line-chart widget per field, updating as new entries arrive.

- Optional: use Matlab Visualizations or a Gauge widget in ThingSpeak Apps for a dashboard-style display, and resize/drag widgets on the Public View for the final dashboard layout.

# 6. Alert Mechanism (Question 5)

Two implementation options were considered:

- Option A — ThingSpeak React + ThingHTTP + Email: a ThingSpeak "React" rule watches Field 1/Field 2 and, when a condition (e.g. Field1 > 38) is true, runs a ThingHTTP action that calls an email service.

- Option B — ESP32-side check + IFTTT Webhooks (used in the provided firmware): the ESP32 evaluates the threshold locally right after each reading and, if crossed, POSTs to an IFTTT Webhooks event, which is configured with an "Email" action (via IFTTT's Gmail/Email service).

Option B was implemented because it reacts immediately (no dependency on ThingSpeak's polling interval) and keeps the alert logic transparent in the firmware:

bool tempAlert = (temperature > TEMP_HIGH_THRESHOLD) ||

(temperature < TEMP_LOW_THRESHOLD);

bool humAlert  = (humidity > HUM_HIGH_THRESHOLD) ||

(humidity < HUM_LOW_THRESHOLD);

if ((tempAlert || humAlert) &&

(millis() - lastAlertTime >= ALERT_COOLDOWN)) {

// POST to IFTTT Webhooks -> triggers email

}

## IFTTT setup

- Create an applet: If Webhooks (event name weather_alert) Then Email (or Gmail).

- Copy the personal Webhooks key from ifttt.com/maker_webhooks/settings into IFTTT_KEY.

- An ALERT_COOLDOWN (5 minutes) prevents inbox flooding while a threshold remains crossed.

# 7. Tools Used

- ESP32 Development Board

- DHT22 Temperature & Humidity Sensor

- Arduino IDE / PlatformIO

- ThingSpeak Cloud Platform

- IFTTT Webhooks (email alert delivery)

# 8. Submission Checklist

- Project Report (this PDF)

- Source Code — weather_station.ino

- Screenshots of ThingSpeak dashboard and triggered alerts

Folder name for Google Drive submission: IoT_Weather_Station_<StudentName>